Innovative Technology Ltd

Vertical Stacker VS1

(for use with the NV4 Smiley Validator)

Technical Manual

Innovative Technology Ltd.

Oldham

England  OL1  4EQ

Tel:	+44-161-626-9999

Fax:	+44-161-620-2090

General Description

The VS1 Vertical Stacking Mechanism is designed to be coupled with the NV4 Smiley Validator . The control software for the Stacker is inbuilt into the NV4 Validator which will automatically detect when a stacker is connected.

The Stacker takes the accepted note from the upper exit point on the Validator.

Pegs then grip the note securely and the note is pulled down onto the note stack and released. The pegs then return to their parking position without disturbing the note stack ready to receive the next note. ( Pat Pending on this functionality )

A cash cover is provided with the unit.

Functional Description

The following is a general description of  how the stacking mechanism functions from being powered up, to stacking a note and subsequently returning to the parking position.

- Whenever power is applied to the unit it performs a series of self tests which will then cycle the stacker and re-park the gripper pegs at the top of the unit.

- Once a note has been accepted by the Validator the note will be transferred from the upper exit position into the awaiting gripper pegs in the stacker.

- The gripper pegs then pull down the note into the stacker.

- A counter-rotating wheel ensures that previously stacked notes are not disturbed when the note is being dragged past the stack.

- The pegs release the note at the bottom of the stack.

- The pegs then start to return to the top of the unit. During this return cycle the pegs tilt back to prevent them from catching on the note stack

- Once the pegs approach the top of the unit the pegs flip forward again and are parked with the pegs open, ready to receive the next note.

- The Stacker detects when the unit is full using optical sensors triggered by a tab on the note clamping plate

Electrical connection

The five pin connector lead provides the direct connection to the NV4 Smiley Validator.

Mechanical Specification

- Will stack notes from the NV4 Validator within the following sizes :-

- Maximum note width 82mm.

- Minimum note width 56mm.

- Maximum note length 160mm.

- Minimum note length 90mm.

- The normal holding capacity is approx. 300 notes. However this capacity can be  increased to 450 notes if the notes are ‘crisp and new’ or reduced to approx. 250 notes if the notes are ‘badly crumpled’.

- Unladen weight of the unit is 1420g.

- The cycle time to stack a note is typically 5 secs.

- The unit is designed to operate with the Validator at the top. It is not designed to work upside down or on its side.

- For dimensions and fixing locations, see drawing GA114 in the Appendix.

- An Exploded view of all components is provided, see GA115 in the Appendix.

- A cash cover is provided . If the customer wishes to make this cover lockable this can be done by adding a standard cam lock , such as :-

Sidlock Ref No C510S , 90 right , Single entry lock with a KS-17 cam.
As supplied by	DZUS Fastener Europe
			Farnham Trading Estate , 
			Farnham , Surrey , England  GU9 9PL
			tel  +44 (0)1252-714422       fax  +44 (0)01252-712738

Electrical Specification

See the NV4 Smiley Validator Technical Manual

The unit meets the requirements of all applicable European EMC regulations.

Environmental Specification

The unit operates over a temperature range of -10C to +50C in a non condensing atmosphere.

Fault Finding

| Stacker does not function | - Ensure the NV4 Smiley Validator has version 2.41 firmware or greater. - Ensure the Validator functions correctly without the stacker attached. - Ensure the 5 pin connector is plugged into the Validator correctly. - Switch the supply to the Validator “off & on”. The stacker should complete a cycle and re-park the Pegs. |
| --- | --- |
| Notes are constantly rejected by the validator | - Check the Validator functions correctly without the stacker attached. |
| Stacker does not put the Validator “out of service” when the Stacker is full. | - Check the connector to the Validator is fitted correctly. - Check this function by blocking the full sensors located at the bottom left hand side. |

Appendix

GA114 Outline dimensions

GA115 Exploded diagram

| Issue No. | Mod No. | Date | Done by |
| --- | --- | --- | --- |
| ISSUE 1 | PROD’ REL | 07-04-99 | AB |
| ISSUE 2 | MOD 255 | 02-06-99 | MKS |
| ISSUE 3 | MOD 309 | 5-11-99 | MKS |