Innovative Technology Ltd

Horizontal Stacker HS1

(for use with the NV4 Smiley Validator)

Technical Manual

Innovative Technology Ltd.

Oldham

England

Tel:	+44-161-626-9999

Fax:	+44-161-620-2090

General Description

The HS1 Horizontal Stacking Mechanism is designed to be coupled with the NV4 Validator. The control software for the Stacker is built into the NV4 Validator which will automatically detect when a Horizontal stacker is connected.

The Horizontal Stacker takes the accepted note from the lower exit point on the Validator. The notes are pushed into the Stacker by the validator into rotating vanes which subsequently push the note into a Lower Holding Chamber. Once this sequence is complete the validator is now ready to accept a new note.

Functional Description

The following is a general description of  how the stacking mechanism functions from being powered up, to stacking a note and subsequently returning to the parking position.

- When power is applied to the unit it performs a series of self tests. These include Parking position of the rotating vanes and a check to see if the stacker is already full or jammed. If the stacker is full or jammed the validator will be inhibited.

- Once the note has been accepted by the validator, it exits through the lower exiting position and is pushed by the validator through into the stacker and rests upon two horizontal stationary vanes.

- Once in position the vanes simultaneously rotate inwards while the puller is quickly pushed down to drag any remaining length of the note into the stacker.

- The vanes complete one full revolution and return to their initial horizontal parking position. This parking position is controlled by optical sensors.

- On completion of the above, a credit vend is given, and the validator is now ready to accept the next note.

- The stacker detects when it is full by the right vane lifting during its rotation. When this happens 5 times the stacker is deemed full and the validator will be inhibited. This position is again governed by optical sensors.

Mechanical Specification

- Will stack notes from the NV4 validator within the following sizes :-

- Maximum note width 82mm.

- Minimum note width 56mm.

- Maximum note length 160mm.

- Minimum note length 90mm.

- The normal holding capacity is approx 240 notes. However this capacity is increased if notes are ‘crisp and new’ or reduced if the notes are ‘badly crumpled’.

- The unladen weight of the stacker with validator is 1600g.

- The cycle time from initial note insertion into validator, to fully stacked is typically 4-5 secs.

- The unit is designed to operate with the validator at the front. It will not work upside down or on its side.

- For dimensions and fixing locations see drawing GA118 in the Appendix.

- For information on the various fixing methods available by either using the standard Bezel refer to GA105 & GA118, while information on using the Universal Bezel & Escutcheon Plate refer to GA127, GA128 & GA118. All drawings can be found in the Appendix.

- An exploded view of all components is provided see GA121 in the Appendix.

- The horizontal stacker unit is Lockable by use of a standard padlock with shackle diameter no greater than 3.2mm.

Electrical Specification

See NV4 Technical Manual

Environmental Specification

The unit operates over a temperature range of -10°C to +50°C in a non condensing atmosphere.

Fault Finding

i)  No power to the stacker unit. Check the power supply to the NV4 validator and that the stacker is connected to the validator.

ii)  Vanes not in correct horizontal parking position. Turn the power supply to the validator Off/On. The stacker should then rotate the vanes to the horizontal parking position.

iii)  Stacker doesn’t seem to communicate with the validator. Check validator has Software version 2.31 or later.

iv)  Various problems can arise if the power supply to the validator unit is not capable of supplying 1 Amp and above.

v)  Stacker does not seem to be positioned securely. Check that stacker is pushed up to the validator and that the stacker locator as been rotated to lock it in position.

(Note! The locator must be rotated out of the way before stacker will ‘Butt Up’ to validator).

Appendix

GA105   Fixing of Standard Bezel.

GA118   Outline Dimensions and Fixing Holes.

GA121 Exploded View of Horizontal Stacker

GA 127. Universal Bezel Fixing Instructions

GA128 Universal Bezel Fixing Instructions

| Issue No. | Mod No. | Date | Done by |
| --- | --- | --- | --- |
| 1 | Prod Rel | 15/03/99 | CLD |