Innovative

Technology Ltd

SSP

Smiley® Secure Protocol

Document Issue  7     Protocol Version 2

COPYRIGHT  2000.  Innovative Technology Ltd.  Derker Street, Derker, Oldham.  OL1 4EQ

# Change History.

| Innovative Technology Ltd |  |  |  |  |
| --- | --- | --- | --- | --- |
| Title: | SSP Gaming Protocol |  |  |  |
| Drawing No: | GA138 |  | Project: |  |
| Author: | P. Dunlop |  | Date: | 26/05/98 |
| Format: | MS Word | 2000 |  |  |
| Issue | Protocol Ver | ReleaseDate | Mod By | Comments |
| Issue 1 | 1 | 26/05/98 | PD |  |
| Issue 2 | 1 | 03/02/99 | TB |  |
| Issue 3 | 1 | 11/06/99 | AK |  |
| Issue 4 | 2 | 4/02/00 | PD |  |
| Issue 5 | 2 | 20/06/00 | PD |  |
| Issue 6 | 2 | 26/10/00 | PD |  |
| Issue 7 | 2 | 20/11/00 | PD |  |

Issue 4. – Peter Dunlop 13/01/2000

Introduction of generic commands.  Introduction of commands/ responses for coin readers and coin hoppers.  Introduction of addressing structure.  Introduction of encrypted packets.  Upgrade “Protocol Version’ to 2.

Issue 5 – Peter Dunlop 20/06/2000

Clarification of remote download specification – addition of example sequences.  Correction of command conflict – SYNC and LAST REJECT CODE specified with same code, LAST REJECT CODE has been changed to 0x17.  Addition of notes to identify function not currently implemented on NV4 / NV4X.

Issue 6. – Peter Dunlop 26/10/00

Block size missing from header description in version 5 – fixed.   Rewording of description of remote downloading protocol.  Addition of maximum block size.  No code changes required in product firmware or demo code.  Changed example CRC code from assember example to C example.  Addition of simplified remote programming flow chart.

Issue 7. – Peter Dunlop 20/11/00

Introduction of basic card reader commands.  Addition of FAIL as a generic response. Addition of manufacturers extension generic command for use internally.

Table Of Contents

Change History.	2

1.0 General Description.	4

2.0 Hardware Layer.	5

Recommended Connectors	5

3.0 Transport Layer.	6

3.1 Packet Format.	6

3.2 Packet Sequencing.	6

4.0 Encryption Layer. (Currently not implemented)	8

4.1 Packet Format.	8

4.2 Encryption Keys.	8

4.3 Encryption Algorithm.	8

4.4 Encryption Example.	9

5.0 Control Layer.	10

5.1 Introduction	10

5.2 Addressing.	10

5.3 Peripheral Validation.	10

5.4 Generic Commands and Responses.	11

5.4.1 Generic Commands	11

5.4.2 Generic Responses	11

5.4.3 Remote Programming (Version 2.65 and later).	12

5.4.4 Example programming file formats (ITL NV4 Validator).	13

5.4.5 Simplified remote programming flow chart.	15

5.5 Banknote Validator	16

5.5.1 BNV Operation.	16

5.5.2 BNV Commands	16

5.5.3 BNV Response To Polls	18

5.6 Coin Acceptor.	19

5.6.1 Coin Acceptor Operation.	19

5.6.2 Coin Acceptor Commands	19

5.6.3 Coin Acceptor Responses to Polls	21

5.7 Coin Hopper	22

5.7.1 Coin Hopper Operation.	22

5.7.2 Coin Hopper Commands.	22

5.7.3 CH Responses to Polls.	22

5.8 Basic Card Reader.	24

5.8.1 Basic Card Reader Operation.	24

5.8.2 Basic Card Reader Commands.	24

5.8.3 Basic Card Reader Responses to Polls.	25

Appendix. B – Block Encryption Routines.	28

B.1 Encryption routine	28

B.2 Decryption routine	28

Appendix. C – CRC Calculation Routines.	29

# 1.0 General Description.

SSP is a secure interface specifically designed to address the problems experienced by cash handling systems in gaming machines.  Problems such as acceptor swapping, reprogramming acceptors and line tapping are all addressed.

The interface uses a master slave model, the host machine is the master and the peripherals (note acceptor, coin acceptor or coin hopper) are the slaves.  Data transfer is over a multi-drop bus using clock asynchronous serial transmission with simple open collector drivers.   The integrity of data transfers is ensured through the use of 16 bit CRC checksums on all packets.  Each SSP device of a particular type has a unique serial number; this number is used to validate each device in the direction of credit transfer before transactions can take place.  To provide extra security the protocol can operate in an encrypted mode to protect the system from fraud through bus monitoring.  To provide this security a constantly changing random 64 bit key is used.  Commands are currently provided for coin acceptors, note acceptors and coin hoppers.  All current features of these devices are supported.

Features:

- Serial control of Note / Coin Validators and Hoppers

- 4 wire (Tx, Rx, +V, Gnd) system

- RS232 (like) - open collector driver

- High Speed 9600 Baud Rate

- 16 bit CRC error checking

- Data Transfer Mode

- 64 Bit Encrypted Mode

- Proven in the field

Benefits:

- Simple and low cost interfacing of transaction peripherals.

- High security control of payout peripherals.

- Defence against surrogate validator fraud.

- Straightforward integration into host machines.

- Remote programming of transaction peripherals

- Open standard for universal use.

Enhancements of SSP can be requested by contacting:

Innovative Technology Ltd

Derker Street

Oldham

OL1 4EQ

# 2.0 Hardware Layer.

Communication by character transmission based on standard 8-bit asynchronous data transfer.  Only four wires are required TxD, RxD, +V and ground. The transmit line of the host is open collector, the receive line of each peripheral has a 10Kohm pull-up to 5 volts.  The transmit output of each slave is open collector, the receive input of the host has a single 3k3 ohm pull-up to 5 volts.

The data format is as follows:

Encoding:	NRZ

Baud Rate:	9600

Duplex:	Full Duplex

Start bits:	1

Data Bits:	8

Parity:		none

Stop bits:	2

Power to peripheral devices would normally be via the serial bus however devices that require a high current supply in excess of 1.5 Amps e.g. hoppers would be expected to be supplied via a separate connector.

### Recommended Connectors

Two types of connectors are recommended the first is a 15 pin 0.1” pitch header (Molex 22-01-2155), this is primarily for use on bank note acceptors.

| Signal | Pin |
| --- | --- |
| TxD | Pin 1 |
| RxD | Pin 5 |
| Address 0 (Currently not implemented) | Pin 6 |
| GND | Pin 12 |
| +12V | Pin 11 |
| ENABLE | Link Pin 3 to Pin 8. |

The second is a 10 pin 0.1” dual row shrouded header with polarized slot.  This is primarily for use with coin acceptors.  The pin out is shown below.

| 1 | TxD | Reserved | 2 |
| --- | --- | --- | --- |
| 3 | RxD | Reserved | 4 |
| 5 | Address 0 | Address 1 | 6 |
| 7 | + 12 Volts | Ground | 8 |
| 9 | Address 2 | Address 4 | 10 |

# 3.0 Transport Layer.

## 3.1 Packet Format.

Data and commands are transported between the host and the slave(s) using a packet format as shown below.

| STX | SEQ/Slave ID | LENGTH | DATA | CRCL | CRCH |
| --- | --- | --- | --- | --- | --- |

STX:			Single byte indicating the start of a message - 0x7F hex.

SEQ/Slave ID:	Bit 7 is the sequence flag of the packet, bits 6-0 represent the 				address of the slave the packet is intended for, highest allowable slave 			ID is 0x7D

LENGTH:		The length of the data included in the packet - this does not include  		STX, the CRC or the slave ID.

Slave ID:		Single byte used to identify the address of the slave the packet is 				intended for.

DATA:		Commands or data to be transferred.

CRCL, CRCH:	Low and high byte of a forward CRC-16 algorithm using the

polynomial (X16 + X15 + X2 +1) calculated on all bytes,

except STX.  It is initialised using the seed 0xFFFF.  The CRC is 				calculated before byte	stuffing.

## 3.2 Packet Sequencing.

Byte stuffing is used to encode any STX bytes that are included in the data to be transmitted.  If 0x7F (STX) appears in the data to be transmitted, then it should be replaced by 0x7F, 0x7F. Byte stuffing is done after the CRC is calculated, the CRC its self can be byte stuffed. The maximum length of data is 0xFF bytes. The sequence flag is used to allow the slave to determine whether a packet is a re-transmission due to its last reply being lost. Each time the master sends a new packet to a slave it alternates the sequence flag. If a slave receives a packet with the same sequence flag as the last one, it does not execute the command but simply repeats its last reply. In a reply packet the address and sequence flag match the command packet. This ensures that no other slaves interpret the reply as a command and informs the master that the correct slave replied.

After the master has sent a command to one of the slaves, it will wait for 1 second for a reply. After that, it will assume the slave did not receive the command intact so it will re-transmit it with the same sequence flag. The host should also record the fact that a gap in transmission has occurred and prepare to poll the slave for its serial number identity following the current message. In this way, the replacement of the hosts validator by a fraudulent unit can be detected. The frequency of polling should be selected to minimise the possibility of swapping a validator between polls.

If the slave had not received the original transmission, it will see the re-transmission as a new command so it will execute it and reply. If the slave had seen the original command but its reply had been corrupted then the slave will ignore the command but repeat its reply. After twenty retries, the master will assume that the slave has crashed.  A slave has no time-out or retry limit. If it receives a lone sync byte part way through receiving a packet it will discard the packet received so far and treat the next byte as an address byte.

# 4.0 Encryption Layer. (Currently not implemented)

## 4.1 Packet Format.

Encrypted data and commands are transported between the host and the slave(s) using the transport mechanism described above, the encrypted information is stored in the data field in the format shown below.

| STX | SEQ/Slave ID | LENGTH | DATA | CRCL | CRCH |
| --- | --- | --- | --- | --- | --- |

| STEX | Encrypted Data |
| --- | --- |

| LENGTH | DATA | NEXT KEY | CRCL | CRCH |
| --- | --- | --- | --- | --- |

STEX:			Single byte indicating the start of an encrypted data block - 0x7E hex.

LENGTH:		The length of the data included in the packet - this does not include  		STEX, the next key or the CRC.

DATA:		Commands or data to be transferred.

NEXT KEY:	The key needed to decrypt the next encrypted packet from the host.

CRCL, CRCH:	Low and high byte of a forward CRC-16 algorithm using the

polynomial (X16 + X15 + X2 +1) calculated on all bytes,

except STEX.  It is initialised using the seed 0xFFFF.

## 4.2 Encryption Keys.

At power up the first encrypted data packet sent to the peripheral will be encrypted using the peripherals serial number (the host requests and stores the serial number of each peripheral when it is installed – see control layer).  After the first transmission data is encrypted/decrypted using the key contained in the last packet, the host generates this randomly.  The peripheral will always reply to an encrypted packet with data encrypted using the same key as the original packet from the host.

After the data has been decrypted the CRC algorithm is preformed on all bytes including DATA, NEXT KEY, CRCL and CRCH.  The result of this calculation will be zero if the data has been decrypted with the correct key.  If the result of this calculation is non-zero then the peripheral should assume that the host did not encrypt the data (transmission errors are detected by the transport layer).  The slave should go out of service until it is reset.

## 4.3 Encryption Algorithm.

The encryption algorithm has a 64-bit key.  This is only a short key but when combined with the mechanism for changing the key in every packet the system provides a high level of security.  Appendix B contains C source code for encryption and decryption. The algorithm will easily translate into assembly code as long as the XOR is an operation.  The routines works on blocks of 16 bytes that are packed into an array of four long integers, the key is 8 bytes long and is packed into an array of 2 long integers.  If the data to be sent is not a multiple of 16 bytes then the remaining bytes are packed out with zeros.

## 4.4 Encryption Example.

Convert key into long integer

| Key() | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Ox67 | Ox45 | Ox23 | Ox01 | OxEF | OxCD | OxAB | Ox89 |
| L_key() | 0 |  |  |  | 1 |  |  |  |
| Value | Ox01234567 |  |  |  | Ox89ABCDEF |  |  |  |

Get data bytes

| Data | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | OxFF | OxEE | OxDD | OxCC | OxBB | OxAA | Ox99 | Ox88 | Ox77 | Ox66 | Ox55 |

Convert data into array of 4 long integers any unused bytes set to Ox00

| Data | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Value | OxCCDDEEFF | Ox8899AABB | Ox00556677 | Ox00000000 |

Pass data through encryption algorithm.

| E_Data | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Value | Ox1500F4F6 | OxF56E7CBA | OxDA441723 | Ox5D2743D2 |

Break up into individual bytes for transmission:

| E_Data | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Ox1500F4F6 |  |  |  | OxF56E7CBA |  |  |  | OxDA441723 |  |  |  | Ox5D2743D2 |  |  |  |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | A | B | C | D | E | F |
| Value | F6 | F4 | 00 | 15 | BA | 7C | 6E | F5 | 23 | 17 | 44 | DA | D2 | 43 | 27 | 5D |

# 5.0 Control Layer.

## 5.1 Introduction

The slave can only respond to requests from the master with an address byte that matches the slaves address, at no time will the slave transmit any data that is not requested by the host.  Any data that is received with an address that does not match the slave’s address will be discarded.  The master will poll each slave at least every 5 seconds. The slave will deem the host to be inactive, if the time between polls is greater than 5 seconds.  If the slave does not receive a poll within 5 seconds it should change to its disabled state.   The minimum time between polls is specified for individual peripherals. Only one command can be sent in any one poll sequence.

## 5.2 Addressing.

The address of a peripheral consists of two parts, the fixed part that determines the type of device and the variable part.  The variable part is used if there is a number of the same type of peripheral in the same machine, for example hoppers.  The variable part of the address can be set in one of two ways.  Firstly it can be programmed to a fixed number using a PC tool, or the peripheral can be programmed to take the rest of the address from external pins on the interface connector (Currently not implemented).

| Slave ID (Hex) | Peripheral |
| --- | --- |
| 0x00 | Note validator 0 |
| 0x01 | Note validator 1 |
| 0x02 | Coin Validator 0 |
| 0x03 | Coin Validator 1 |
| 0x04 | Card Reader 0 |
| 0x05 | Card Reader 1 |
| 0x07 | Audit Device |
| 0x08 – 0x0F | Reserved |
| 0x10 – 0x1F | Coin Hoppers 0 – 15 |
| 0x20 – 0x2F | Note Dispensers 0 – 15 |
| 0x30 – 0x3F | Card Dispensers 0 – 15 |
| 0x40 – 0x4F | Ticket Dispensers 0 – 15 |
| 0x50- 0x7E | Unallocated |

## 5.3 Peripheral Validation.

To ensure that credit transfers are only received from or sent to genuine devices, the device receiving the credit must first request the serial number from the sending device and only accept if the serial number matches a pre-programmed number.    The serial number should be requested after each reset and also after each break in communications.  For example a host machine should request a coin acceptors serial number at reset or if a poll sequence is unanswered, before enabling the device.  Also, a coin hopper should not process any dispense commands until the host machine has sent its serial number.

## 5.4 Generic Commands and Responses.

Generic commands are a set of commands that every peripheral must understand and act on.

### 5.4.1 Generic Commands

| Action | Command code (HEX) |
| --- | --- |
| Reset | 0x01 |
| Host Protocol Version | 0x06 |
| Poll | 0x07 |
| Get Serial Number | 0x0C |
| Synchronisation command | 0x11 |
| Disable | 0x09 |
| Enable | 0x0A |
| Program Firmware / currency | 0x0B, Programming Type |
| Manufactures Extension | 0x30, Command, Data |

Reset:  Single byte command, causes the slave to reset.

Host Protocol Version:  Dual byte command, the first byte is the command, the second byte is the version of the protocol that is implemented on the host, current version is 02.

Poll:  Single byte command, no action taken except to report latest events.

Get Serial Number: Single byte command, used to request the slave serial number. Returns 4-byte long integer.

Most significant byte first e.g.

Serial number  = 01873452   = 0x1C962C

So response data would be     0x00 0x1C 0x96 0x2C

Sync:  Single byte command, which will reset the validator to expect the next sequence ID to be 0.

Disable:  Single byte command, the peripheral will switch to its disabled state, it will not execute any more commands or perform any actions until enabled, any poll commands will report disabled.

Enable:  Single byte command, the peripheral will return to service.

Program Firmware / currency: See section 5.4.3 – Remote Programming.

Manufactures Extension: This command allows the manufacturer of a peripheral to send commands specific to their unit.  The intention is that the manufacturer only uses the extension command internally; it should not when operating in a host machine. The specific command and any data for that command should follow the Extension command.

### 5.4.2 Generic Responses

| Generic Response | Response code |
| --- | --- |
| OK | 0xFO |
| Slave Reset | 0xF1 |
| Command not known | 0xF2 |
| Wrong number of parameters | 0xF3 |
| Parameter out of range | 0xF4 |
| Command cannot be processed | 0xF5 |
| Software Error | 0xF6 |
| FAIL | 0xF8 |

OK: Returned when a command from the host is understood and has been, or is in the process of, being executed.

Slave Reset:  Returned when a peripheral has just powered up or when the host has sent a reset command.

Command Not Known: Returned when an invalid command is received by a peripheral.

Wrong Number Of Parameters:  A command was received by a peripheral, but an incorrect number of parameters were received.

Parameter Out Of Range: One of the parameters sent with a command is out of range. E.g. trying to change the route map for channel 34 on a coin acceptor.

Command Cannot Be Processed: A command sent could not be processed at that time.  E.g. sending a dispense command before the last dispense operation has completed.

Software Error: Reported for errors in the execution of software e.g. Divide by zero. This may also be reported if there is a problem resulting from a failed remote firmware upgrade, in this case the firmware upgrade should be redone.

### 5.4.3 Remote Programming (Version 2.65 and later).

Code Summary

| Code | Description |
| --- | --- |
| 0x0B, Type | Start Programming, type (00 – firmware, 01 - currency) |
| 0x16 | Programming Status. |

Using the command 0x0B followed by a parameter that indicates the type of programming required performs remote programming. Send 0x00 for firmware programming and 0x01 for currency data programming.  The peripheral will respond with a generic reply. If the reply is OK, the host should then send the first block of the data file (the file header).  The header has the format shown below. The block size depends on the peripheral used but must be a minimum of 10 bytes to contain the header data.  When the block size for a peripheral is greater than the header length (11 bytes) then the header is padded out with 0’s to the length of a block.  The maximum length of a block is 236 bytes.

| File offset | Description | Size |
| --- | --- | --- |
| 0 | Number of blocks to send (low byte, high byte), including header block | 2 bytes |
| 2 | Manufacture code (of file) e.g. ‘ITL’ | 3 bytes |
| 5 | File type – 0x00 firmware, 0x01 currency | 1 byte |
| 6 | Unit subtype | 1 byte |
| 7 | Unit version | 1 byte |
| 8 | Block length (BL) | 1 byte |
| 9 | Checksum (CRC of data section of file) CRC low byte | 1 byte |
| A | Checksum (CRC of data section of file) CRC high byte | 1 byte |
| B | Padded 0’s to block size | BL-11 bytes |

The peripheral will then respond with OK or HEADER_FAIL depending on the acceptability of this file.

| Response | Code |
| --- | --- |
| OK | 0xF0 |
| HEADER_FAIL | 0xF9 |

The host will then send the required number of data blocks.  The peripheral will respond with a generic response when each packet has been processed.  If the host receives any response other than an OK then that packet is retried three times before aborting the programming (the peripheral should then be reset).  After the last data packet has been sent and a response received, the host will send a programming status command 0x16.  The peripheral will respond with one of the following codes:

| Response | Code |
| --- | --- |
| OK | 0xF0 |
| Checksum Error | 0xF7 |
| FAIL | 0xF8 |

After a successful programming cycle, the peripheral should be reset.  If the programming cycle does not complete successfully, then the peripheral should be disabled until it can be programmed successfully.  In the case of an unsuccessful firmware programming cycle, the new firmware will either be discarded or partly programmed.  If the firmware has been partly programmed, then the peripheral will respond to all Polls with the generic response ‘Software Error’. The peripheral will not allow the host to enable it until it receives a complete and valid firmware file.

### 5.4.4 Example programming file formats (ITL NV4 Validator).

Programming files supplied by Innovative Technology Ltd for NV4 BNV are formatted as follows: Variable number of blocks depending on currency, fixed block length (128 bytes).

Currency File Example:

| Header block | 92, 01, 49, 54, 4C, 01, 01, 01, 80, 8D, 2C | Padded to block length with 0’s |
| --- | --- | --- |
| 1st data block | F0, 34, C0, 21, D3, 00, 00, 5F, 5F, ……. | 80h data bytes |
| 2nd data block | FF, 24, D3, 21, 45, 01, 00, 3F, 5F, ……. | 80h data bytes |
|  | . |  |
|  | . |  |
| nth data block | FF, 24, D3, 21, 45, 01, 00, 3F, 5F, ……. | 80h data bytes |

Firmware File Example:

| Header block | 01, 01, 49, 54, 4C, 00, 01, 01, 80, FD, 22 | Padded to block length with 0’s |
| --- | --- | --- |
| 1st data block | FF, 24, D3, 21, 45, 01, 00, 3F, 5F,……… | 80h data bytes |
| 2nd data block | F0, 34, C0, D1, D3, 00, 00, 5F, 5F, ……. | 80h data bytes |
|  | . |  |
|  | . |  |
| 257th data block | FF, 24, D3, 21, 45, 01, 00, 3F, 5F, ……. | 80h data bytes |

### 5.4.5 Simplified remote programming flow chart.

Open File

Read Header

Determine: download TYPE, block size

and number of blocks

Send:

START_PROGRAMING, TYPE

Send: HEADER

(first BL bytes of file)

OK?

FAIL

Send next data block

(next BL bytes of file)

Last Data Block?

Send:

PROG_STATUS

OK?

FAIL

Send: RESET

DONE

## 5.5 Banknote Validator

### 5.5.1 BNV Operation.

When the validator has recognised a note, it will not start to stack it until it receives the next valid poll command after the read n (n<>0) has been sent.  The note will be rejected if the host responds with a REJECT.

### 5.5.2 BNV Commands

| Action | Command code (HEX) |
| --- | --- |
| Set inhibits | 0x02 |
| Display on | 0x03 |
| Display Off | 0x04 |
| Set-up Request | 0x05 |
| Reject | 0x08 |
| Unit data | 0x0D |
| Channel Value data | 0x0E |
| Channel Security data | 0x0F |
| Channel Re-teach data | 0x10 |
| Last Reject Code | 0x17 |

Set Inhibits: Variable length command, used to control which channels are enabled. The command byte is followed by 2 data bytes, these bytes are combined to create the INHIBIT_REGISTER, each bit represents the state of a channel (LSB= channel 1, 1=enabled, 0=disabled). At power up all channels are inhibited and the validator is disabled.

Display On: Single Byte command, turns on the display illumination bulb.

Display Off: Single Byte command, turns off the display illumination bulb.

Reject: Single byte command causing the validator to reject the current note.

Set-up Request: Single byte command, used to request information about a slave.  Slave will return the following data: Unit Type, Firmware version, Country Code, Value multiplier, Number of channels,(if number of channels is 0 then 0 is returned and next two parameters are not returned) Value per channel, security of channel, Reteach count, Version of Protocol.

Response to Set-up request:

| Data | Size/type | Notes |
| --- | --- | --- |
| Unit Type | 1 byte, integer | 0x00 Note Validator |
| Firmware Version | 4 bytes, string | XX.XX (can include space) |
| Country Code | 3 bytes, string | See Country Code Table |
| Value Multiplier | 3 bytes, integer | 24 bit value |
| Number of channels | 1 byte, integer | Highest used channel |
| Channel Value | 15 bytes, integer | bytes 1 – 15 values |
| Security of Channel | 15 bytes, integer | bytes 1 – 15 security |
| Reteach count | 3 byte, integer | Byte 1 - reteach count. Byte 2, 3 flag register indicating which channels have been modified. All set to zero at factory. |
| Protocol version | 1 byte, integer |  |

Unit Data Request: Single byte command which returns, Unit type (1 Byte integer), Firmware Version (4 bytes ASCII string), Country Code (3 Bytes ASCII string), Value Multiplier (3 bytes integer), Protocol Version (1 Byte, integer)

Channel Value Request:  Single byte command which returns a number of channels byte    (the highest channel used) and then 1 to n bytes which give the value of each channel up to the highest one, a zero indicates that the channel is not implemented.

e.g. A validator has notes in Channels 1,2,4,6,7 so this command would return 07,01,02,00,04,00,06,07.  (The values are just examples and would depend on the currency of the unit).

The actual value of a note is calculated by multiplying the value multiplier by channel value.

If the number of channels is 0 then only one 0 will be returned.

Channel Security Data:  Single byte command which returns a number of channels byte (the highest channel used) and then 1 to n bytes which give the security of each channel up to the highest one, a zero indicates that the channel is not implemented.

(1 = low, 2 = std, 3 = high,   4 = inhibited).

e.g.  A validator has notes in Channels 1,2,4,6,7  channel 1 is low security, channel 6 is high security , all the rest are standard security.

The return bytes would be

07,01,02,00,02,00,02,03

If the number of channels is 0 then only one 0 will be returned.

Channel Reteach Data: Single byte command, which returns 3 bytes.  First byte - the number of times the unit has been manually taught. (1 for each face). Second byte - Channels 1 to 8 flag register  bit 0 = channel 1 to bit 7 = channel 8 if set shows that the indicated channel has been altered. Third byte is as second but the channels shown are bit 1 = channel 9 to bit 6 = channel 15.

Last Reject Code: Single byte command, which will return a single byte that indicates the reason for the last reject.  The codes are shown in the table below.  Specifics of note validation not shown to protect integrity of manufacturers security (Version 2.66 and later).

| Code | Reject Reason |
| --- | --- |
| 0x00 | Note Accepted |
| 0x01 | Note length incorrect |
| 0x02 | Reject reason 2 |
| 0x03 | Reject reason 3 |
| 0x04 | Reject reason 4 |
| 0x05 | Reject reason 5 |
| 0x06 | Channel Inhibited |
| 0x07 | Second Note Inserted |
| 0x08 | Reject reason 8 |
| 0x09 | Note recognised in more than one channel |
| 0x0A | Reject reason 10 |
| 0x0B | Note too long |
| 0x0C | Reject reason 12 |
| 0x0D | Mechanism Slow / Stalled |
| 0x0E | Striming Attempt |
| 0x0F | Fraud Channel Reject |
| 0x10 | No Notes Inserted |

### 5.5.3 BNV Response To Polls

In response to any command from the master, the slave will respond with a generic response and some data.  If the command is a poll then a message containing a list of events that have occurred since the last poll, each event can only occur once in each response packet.

| Event/ State | Event Code |
| --- | --- |
| Read, n | 0xEF, Channel No |
| Credit, n | 0xEE, CHANNEL No |
| Rejecting | 0xED |
| Rejected | 0xEC |
| Stacking | 0xCC |
| Stacked | 0xEB |
| Safe Jam | 0xEA |
| Unsafe Jam | 0xE9 |
| Disabled | 0xE8 |
| Fraud Attempt, n | 0xE6, Channel No |
| Stacker Full | 0xE7 |

Read:  The slave is reading a note, the second byte indicates which channel the note belongs to, if the channel is currently unknown then zero is returned.

Credit: The slave has accepted currency on the channel indicated, the currency is now past the point where the customer can recover the currency. The credit event is only sent once (except where communication fails).  The second byte indicates the channel of the credit.

Rejecting:  The validator is currently rejecting a note.

Rejected: The slave has rejected the currency that was entered.

Stacking: The slave is moving the currency to a secure location.

Stacked:  The stacking unit has completed it cycle.

Safe Jam:  The slave has jammed and cannot return to service, the user cannot retrieve a note and a credit has been given.

Unsafe Jam:  The slave is jammed and cannot return to service, the credit has not been given and the user may be able to retrieve the note.

Disabled:  The slave has been disabled, either by disabling all channels or the 5 second poll time out has expired.

Fraud Attempt:  The validator has detected an attempt to fish notes out of the Stacker.

## 5.6 Coin Acceptor.

### 5.6.1 Coin Acceptor Operation.

### 5.6.2 Coin Acceptor Commands

| Action | Command code (HEX) |
| --- | --- |
| Set inhibits | 0x02 |
| Set-up Request | 0x05 |
| Unit data | 0x0D |
| Channel Value data | 0x0E |
| Channel Security data | 0x0F |
| Channel Re-teach data | 0x10 |
| Last Reject Code | 0x17 |
| Update Coin Route | 0x12 |

Set Inhibits: Variable length command, used to control which channels are enabled. The command byte is followed by n data bytes, these bytes are combined to create the INHIBIT_REGISTER, each bit represents the state of a channel (LSB= channel 1, 1=enabled, 0=disabled). At power up all channels are inhibited and the validator is disabled.

Set-up Request: Single byte command, used to request information about a slave.  Slave will return the following data: Unit Type, Firmware version, Country Code, Value multiplier, Number of channels,(if number of channels is 0 then 0 is returned and next two parameters are not returned) Value per channel, security of channel, Reteach count, Version of Protocol.

Response to Set-up request:

| Data | Size/type | Notes |
| --- | --- | --- |
| Unit Type | 1 byte, integer | 0x01 Coin Validator |
| Firmware Version | 4 bytes, string | XX.XX (can include space) |
| Country Code | 3 bytes, string | See Country Code Table |
| Value Multiplier | 3 bytes, integer | 24 bit value |
| Number of channels | 1 byte, integer | Highest used channel |
| Channel Value | 15 byte, integer | bytes 1 - n values |
| Security of Channel | 15 byte, integer | bytes 1- n security |
| Reteach count | 3 byte, integer | Byte 1 - reteach count. Byte 2,3 flag register indicating which channels have been modified. All set to zero at factory. |
| Protocol version | 1 byte, integer |  |

Unit Data Request: Single byte command which returns,  Unit type (1 Byte integer), Firmware Version (4 bytes ASCII string), Country Code (3 Bytes ASCII string), Value Multiplier (3 bytes integer), Protocol Version (1 Byte, integer)

Channel Value Request:  Single byte command which returns a number of channels byte    (the highest channel used) and then 1 to n bytes which give the value of each channel up to the highest one, a zero indicates that the channel is not implemented.

e.g. A validator has coins in Channels 1,2,4,6,7 so this command would return 07,01,02,00,04,00,06,07.  (The values are just examples and would depend on the currency of the unit).

The actual value of a coin is calculated by multiplying the value multiplier by channel value.

If the number of channels is 0 then only one 0 will be returned.

Channel Security Data:  Single byte command which returns a number of channels byte (the highest channel used) and then 1 to n bytes which give the security of each channel up to the highest one, a zero indicates that the channel is not implemented.

(1 = low, 2 = std, 3 = high, 4 = inhibited).

e.g.  A validator has coins in Channels 1,2,4,6,7  channel 1 is low security, channel 6 is high security , all the rest are standard security.

The return bytes would be

07,01,02,00,02,00,02,03

If the number of channels is 0 then only one 0 will be returned.

Channel Reteach Data: Single byte command, which returns 3 bytes.  First byte - the number of times the unit has been manually taught. (1 for each face). Second byte - Channels 1 to 8 flag register bit 0 = channel 1 to bit 7 = channel 8 if set shows that the indicated channel has been altered. Third byte is as second but the channels shown are bit 1 = channel 9 to bit 6 = channel 15.

Last Reject Code: Single byte command, which will return a single byte that indicates the reason for the last reject.  The codes are shown in the table below.

Reject Reason Codes.

| Code | Reject Reason |
| --- | --- |
| 0x00 | Coin Accepted |
| 0x01 | Reject error 1 |
| 0x02 | Reject error 2 |
| 0x03 | Reject error 3 |
| 0x04 | Reject error 4 |
| 0x05 | Channel Inhibited (software) |
| 0x06 | Channel Inhibited (SSP) |
| 0x07 | Closely following coin |
| 0x08 | Reject error 8 |
| 0x09 | Match in multiple windows |
| 0x0A | Reject error 10 |
| 0x0B | Reject error 11 |
| 0x0C | Reject error 12 |
| 0x0D | Reject error 13 |
| 0x0E | Strim Attempt |
| 0x0F | Fraud Channel Reject |
| 0x10 –0xFF | Reserved |

Update Coin Route: This command consists of three bytes; the actual command, the channel to be updated and the route information.  The route information is packed into one byte, each bit represents a route, if a bit is set then the route is allowed for that channel.

Example:  0x12, 0x02, 0x24 – Update route for channel 2 to 00100100 (routes 3 and 6).

### 5.6.3 Coin Acceptor Responses to Polls

In response to any command from the master the acceptor will respond with a generic response and some data.  If the command is a poll then a message containing a list of events that have occurred since the last poll, each event can only occur once in each response packet.

| Event/ State | Event Code |
| --- | --- |
| Credit, n | 0xEE, CHANNEL No |
| Rejected | 0xEC |
| Coin Routed | 0xE5, route No |
| Safe Jam | 0xEA |
| Unsafe Jam | 0xE9 |
| Disabled | 0xE8 |
| Fraud Attempt | 0xE6 |

Credit: The slave has accepted currency on the channel indicated, the currency is now past the point where the customer can recover the currency. The credit event is only sent once (except where communication fails).  The second byte indicates the channel of the credit.

Rejected: The slave has rejected the currency that was entered.

Coin Routed:  The acceptor has routed the last coin accepted.

Safe Jam:  The slave has jammed and cannot return to service, the user cannot retrieve a coin and a credit has been given.

Unsafe Jam:  The slave is jammed and cannot return to service, the credit has not been given and the user may be able to retrieve the coin.

Disabled:  The slave has been disabled, either by disabling all channels or the 5 second poll time out has expired.

Fraud Attempt:  The validator has detected an attempt to fish coins out of the unit.

## 5.7 Coin Hopper

### 5.7.1 Coin Hopper Operation.

All transactions with a hopper should be encrypted to prevent dispense commands being recorded and replayed by an external device.  To enable the dispense command of the hopper the host must send it a serial number, which matches the one stored in the hopper.  This has two implications for the hopper firstly it must have some non-volatile memory to store the correct serial number in.  Secondly, it must have a means of switching mode so that the next serial number received is stored as its reference number.  This allows the hopper to be installed in a host machine without manually programming the reference serial number.  As the hopper is in a secure location in the host, this could be done by pressing a recessed button in the hopper.

### 5.7.2 Coin Hopper Commands.

| Action | Command code (HEX) |
| --- | --- |
| Dispense | 0x13, No of coins |
| Host Serial Number Request | 0x14, Serial No |
| Set-up Request | 0x15 |

Dispense:  Two-byte command, the first byte is the command it’s self and the second is the number of coins to dispense.

Host serial number request:  This allows the host machine to send its serial number to the hopper.  After a reset or break in communications the hopper will not process any other commands until it has received a serial number equal to the one it has in its memory.  If more than ten invalid serial numbers are received, the hopper will go out of service until its set-up input is activated.

Set-up Request: Single byte command, used to request information about a slave.  Slave will return the following data: Unit Type, Firmware version, Country Code, Coin Type, Maximum Capacity, Version of Protocol.

Response to Set-up request:

| Data | Size/type | Notes |
| --- | --- | --- |
| Unit Type | 1 byte, integer | 0x02 Coin Hopper |
| Firmware Version | 4 bytes, string | XX.XX (can include space) |
| Country Code | 3 bytes, string | See Country Code Table |
| Coin Type | 1 byte, integer |  |
| Maximum Capacity | 2 Bytes, integer |  |
| Low coin value | 1 Byte, integer | Number of coins left at low coin event. |
| Protocol version | 1 byte, integer |  |

### 5.7.3 CH Responses to Polls.

| Event/ State | Event Code |
| --- | --- |
| Dispensing | 0xD1, No of coins dispensed |
| Dispensed | 0xD2, No of coins |
| Coins Low | 0xD3 |
| Empty | 0xD4 |
| Jammed | 0xD5 |
| Fraud Attempt | 0xE6, Fraud Code |

Dispensing: Two-byte response the second byte is the number of coins that have been dispensed at the point when the poll was received.

Dispensed: Two-byte response that indicates when the hopper has finished a dispense operation, either because the required number of coins have been dispensed or the hopper is out of coins or jammed.   The second byte is the number of coins that were successfully dispensed.

Coins Low: This is reported when the hopper has become low on coins, the hopper will report this event until it is empty or refilled.

Coins Empty:  Single byte response indicating that the hopper is empty of coins; the hopper will report this state until it is filled it will also become disabled.

Jammed: Single byte response that indicates that the hopper is jammed; this is reported until it is unjamed or reset.  It will also become disabled.

## 5.8 Basic Card Reader.

### 5.8.1 Basic Card Reader Operation.

The basic card reader will allow a small amount of data to be securely stored on a removable card.

The reader / card combination provides the following features:

- Each card reports a unique 32-bit ID number.

- A small amount of data can be stored on each card (CS1 16 bytes).

- The host must provide a 32-bit password to access the card.

- The card provides a 24-bit password to the host.

- The host can display messages on the readers display.

Before the card will reports its ID number or allow access to the data on the card the card must be authorised.  The process to do this involves two steps; firstly the host must provide the reader with the correct password for the card, the card will then provide its ID number and a system password.  The host must check that the password provided by the card is correct for this system.  Once this process is complete then the host can read or write data from the card.  The states of the card reader and transitions between states are shown below.

Enable

DISABLED				IDLE

Disable

Card Removed

Card Entered

Card Removed

Authorise

CARD INSERTED				CARD AUTHORISED

Read / Write Card

### 5.8.2 Basic Card Reader Commands.

| Action | Command code (HEX) |
| --- | --- |
| Authorise Card | 0x20, Card Password |
| Read Card | 0x21 |
| Write Card | 0x22, Data |
| End Session | 0x23 |
| Display Message | 0x24, Message, 0x00 |
| Set-up Request | 0x15 |

Authorise Card: Used to authorise a card, this must be successfully completed before the card can be read from or written to.  The command should be followed by a four-byte card password. If the card accepts the password then the reader will respond with OK, a four byte card ID and a three-byte system password, otherwise it will respond with FAIL.  If the returned system password is acceptable to the host then the card is accepted, if not then the host should use the END SESSION command to reject the card.

| Response | Code |
| --- | --- |
| Password Accepted – OK | 0xF0,Card ID (4), System Password (3) |
| Password Rejected – FAIL | 0xF8 |

Read Card: This command returns all the data from the card; the Storage Capacity field of the Set-up Request response determines the number of bytes returned.  This command can only be used after a successful Authorise Card sequence.

Write Card: This command is used to write data to the card, the data to be written to the card should follow the command.  The reader will respond with a generic response; OK if the data was written successfully and FAIL if there was a problem.  This command can only be used after a successful Authorise Card sequence.

End Session: This command is used to end the session with the card after reading and writing to the card, or if the card ID or system password are unacceptable.

Display Message: The host can use this command to display messages on the reader display.  The command should be followed by a NULL terminated string to be displayed. The message will be displayed once only.  If a previous message is still being displayed then the new message will be displayed once the previous one has finished.  The maximum message length is 240 characters.

Set-up Request: Single byte command, used to request information about a slave.  Slave will return the following data: Unit Type, Firmware version, Storage Capacity, Unit Capability, and Version of Protocol.

Response to Set-up request:

| Data | Size/type | Notes |
| --- | --- | --- |
| Unit Type | 1 byte, integer | 0x03 - Basic Card Reader |
| Firmware Version | 4 bytes, string | XX.XX (can include space) |
| Storage Capacity | 1 byte, integer | Number of bytes (CS1 – 16) |
| Units Capability | 1 byte, integer | 0x00 Read only, 0x01 W/R |
| Protocol version | 1 byte, integer |  |

### 5.8.3 Basic Card Reader Responses to Polls.

| Event/ State | Event Code |
| --- | --- |
| Card Inserted | 0xC0 |
| Card Authorised | 0xC1, Pasword |
| Card Removed | 0xC2 |
| Disabled | 0xE8 |
| Unit Fault | 0xC3, Fault Code |

Card Inserted: This state is entered when a card is entered into the reader; it can only be entered from the idle state. The state is left when the card is authorised or when the card is removed

Card Authorised: This state is entered after a successful card authorise sequence. This is the only state in which the card can be read from and written to.  The state is exited after the host issues a End Session command (returning to the Card Inserted state), or if the card is removed (returning to the idle state after reporting a Card Removed event).

Card Removed: This event is reported when the card is removed from the reader, the reader will return to the idle state.

Disabled:  This state is the initial state after power up; it is also entered after the host issues a Disable command.  The display is off, and all card entered are ignored.

Unit Fault:

Appendix A – Country Codes.

| Country | Abbr. | Country | Abbr. |
| --- | --- | --- | --- |
| Algeria | DZD | Japan | JPY |
| Arab (Un. Emir) | AED | Jordan | JOD |
| Argentina | ARA | Kenya | KES |
| Australia | AUD | Kuwait | KWD |
| Austria | ATS | Lebanon | LBP |
| Bahrain | BHD | Luxembourg | LUF |
| Belgium | BEF | Malaysia | MYR |
| Brazil | BRE | Mexico | MXP |
| Bulgaria | BGL | Morocco | MAD |
| Canada | CAD | Netherlands | NLG |
| China | CYN | Netherlands Ant. | ANG |
| C.I.S | RBL | New Zealand | NZD |
| Columbia | COP |  |  |
| Cuba | CUP | Nigeria | NGN |
| Cyprus | CPY | Norway | NOK |
| Czechoslovakia | CSK | Oman | OMR |
| Denmark | DKK | Philippines | PHP |
| Egypt | EGP | Poland | PLZ |
| Finland | FIM | Portugal | PTE |
| France | FRF | Qatar | QAR |
| Germany | DEM | Rep. of Croatia | HRD |
| Great Britain | GBP | Rep. of Slovenia | SIT |
| Greece | GRD | Rumania | ROL |
| Hong Kong | HKD | Saudi Arabia | SAR |
| Hungary | HUF | Singapore | SGD |
| Iceland | ISK | South Africa | ZAR |
| India | INR | Spain | ESP |
| Indonesia | IDR | Sri Lanka | LKR |
| Iran | IRR | Sweden | SEK |
| Iraq | IQD | Switzerland | CHF |
| Ireland | IEP | Syria | SYP |
| Israel | ILS | Tunisia | TND |
| Italy | ITL | Turkey | TRL |
|  |  | United States | USD |

# Appendix. B – Block Encryption Routines.

## B.1 Encryption routine

/***********************************************************************

Function CODE

Function to encrypt 16 bytes of data (4 long ints)

Usage:   :     	code (long_data, long_current_key);

Parameters:	data()  - 16 bytes to be encrypted packed into 4 long ints

Key()  - key to use packed into array of 2 long ints

Returns:  	data() - 16 bytes of encrypted data packed into 4 long ints

Locals:	w,x,y,z,delta,n

/***********************************************************************

void code(long* data, long* key)  {

unsigned long w=data[0],x=data[1],y=data[2],z=data[3], sum=0,   /* set up */

delta=0x9e3779b9, n=32 ;             /* a key schedule constant */

while (n-->0) {                       	/* basic cycle start */

sum += delta ;

w += (z<<4)+key[0] ^ z+sum ^ (z>>5)+key[1] ;

x += (w<<4)+key[1] ^ w+sum ^ (w>>5)+key[0] ;

y += (x<<4)+key[0] ^ x+sum ^ (x>>5)+key[1] ;

z += (y<<4)+key[1] ^ y+sum ^ (y>>5)+key[0] ;

}                    				/* end cycle */

data0]=w ; data[1]=x ; data[2]=y ;data[3]=z;

}

## B.2 Decryption routine

/***********************************************************************

Function CODE

Function to encrypt 16 bytes of data (4 long ints)

Usage:   :     	code (long_data, long_current_key);

Parameters:	data() - 16 bytes of encrypted data packed into 4 long ints

Key()  - key to use packed into array of 2 long ints

Returns:       	data()  - 16 bytes of decrypted data packed into 4 long ints

Locals:	w,x,y,z,delta,n

/***********************************************************************

void decode(long* data, long* key)  {

unsigned long n=32, sum,w=data[0],x=data[1], y=data[2], z=data[3],

delta=0x9e3779b9 ;

sum=delta<<5 ;

while (n-->0) {    	  /* start cycle */

z-= (y<<4)+key[1] ^ y+sum ^ (y>>5)+key[0] ;

y-= (x<<4)+key[0] ^ x+sum ^ (x>>5)+key[1] ;

x-= (w<<4)+key[1] ^ w+sum ^ (w>>5)+key[0] ;

w-= (z<<4)+key[0] ^ z+sum ^ (z>>5)+key[1] ;

sum-=delta ;

}                  			 /* end cycle */

data[0]=w ; data[1]=x ; data[2]=y ;data[3]=z;

}

# Appendix. C – CRC Calculation Routines.

/*

|-----------------------------------------------------------------------|

|       c INNOVATIVE TECHNOLOGY LTD 2000  	  |

|-----------------------------------------------------------------------|

| TITLE  :CRC Functions    |Language : C         	  |

| DRG No :      	        |Project  :          		  |

|Author  : P Dunlop             |Date     :17-04-2000       |

|Revision: A                         |                                    |

|-----------------------------------------------------------------------|

| Issue No.  |    Mod No.  |      DATE  |      Mod By.    |

|-----------------------------------------------------------------------|

|   ISSUE A  |                   |                  |                     |

|-----------------------------------------------------------------------|

|External files Used:            			              |

|   Includes:              				              |

|   Linked:                                                        	   |

|   Config:              				              |

|   Docs:                					  |

|-----------------------------------------------------------------------|

|  CRC FUNCTIONS FOR POLYNOMIAL (X^16)+(X^15)+(X^2)+1 AS USED IN SSP    |

|  CRC IS INITIALISED WITH THE SEED 0xFFFF                                |

|------------------------------------------------------------------------

*/

#define FALSE           0x00

#define TRUE            0x01

unsigned char CRCL,CRCH;

int CRC_Table[8*32]={

0x0000,0x8005,0x800F,0x000A,0x801B,0x001E,0x0014,0x8011,

0x8033,0x0036,0x003C,0x8039,0x0028,0x802D,0x8027,0x0022,

0x8063,0x0066,0x006C,0x8069,0x0078,0x807D,0x8077,0x0072,

0x0050,0x8055,0x805F,0x005A,0x804B,0x004E,0x0044,0x8041,

0x80C3,0x00C6,0x00CC,0x80C9,0x00D8,0x80DD,0x80D7,0x00D2,

0x00F0,0x80F5,0x80FF,0x00FA,0x80EB,0x00EE,0x00E4,0x80E1,

0x00A0,0x80A5,0x80AF,0x00AA,0x80BB,0x00BE,0x00B4,0x80B1,

0x8093,0x0096,0x009C,0x8099,0x0088,0x808D,0x8087,0x0082,

0x8183,0x0186,0x018C,0x8189,0x0198,0x819D,0x8197,0x0192,

0x01B0,0x81B5,0x81BF,0x01BA,0x81AB,0x01AE,0x01A4,0x81A1,

0x01E0,0x81E5,0x81EF,0x01EA,0x81FB,0x01FE,0x01F4,0x81F1,

0x81D3,0x01D6,0x01DC,0x81D9,0x01C8,0x81CD,0x81C7,0x01C2,

0x0140,0x8145,0x814F,0x014A,0x815B,0x015E,0x0154,0x8151,

0x8173,0x0176,0x017C,0x8179,0x0168,0x816D,0x8167,0x0162,

0x8123,0x0126,0x012C,0x8129,0x0138,0x813D,0x8137,0x0132,

0x0110,0x8115,0x811F,0x011A,0x810B,0x010E,0x0104,0x8101,

0x8303,0x0306,0x030C,0x8309,0x0318,0x831D,0x8317,0x0312,

0x0330,0x8335,0x833F,0x033A,0x832B,0x032E,0x0324,0x8321,

0x0360,0x8365,0x836F,0x036A,0x837B,0x037E,0x0374,0x8371,

0x8353,0x0356,0x035C,0x8359,0x0348,0x834D,0x8347,0x0342,

0x03C0,0x83C5,0x83CF,0x03CA,0x83DB,0x03DE,0x03D4,0x83D1,

0x83F3,0x03F6,0x03FC,0x83F9,0x03E8,0x83ED,0x83E7,0x03E2,

0x83A3,0x03A6,0x03AC,0x83A9,0x03B8,0x83BD,0x83B7,0x03B2,

0x0390,0x8395,0x839F,0x039A,0x838B,0x038E,0x0384,0x8381,

0x0280,0x8285,0x828F,0x028A,0x829B,0x029E,0x0294,0x8291,

0x82B3,0x02B6,0x02BC,0x82B9,0x02A8,0x82AD,0x82A7,0x02A2,

0x82E3,0x02E6,0x02EC,0x82E9,0x02F8,0x82FD,0x82F7,0x02F2,

0x02D0,0x82D5,0x82DF,0x02DA,0x82CB,0x02CE,0x02C4,0x82C1,

0x8243,0x0246,0x024C,0x8249,0x0258,0x825D,0x8257,0x0252,

0x0270,0x8275,0x827F,0x027A,0x826B,0x026E,0x0264,0x8261,

0x0220,0x8225,0x822F,0x022A,0x823B,0x023E,0x0234,0x8231,

0x8213,0x0216,0x021C,0x8219,0x0208,0x820D,0x8207,0x0202};

//------------------------------------------------------------------------

void Update_CRC(unsigned char num){

unsigned int  table_addr;

table_addr=(num ^ CRCH);

CRCH=(CRC_Table[table_addr] >> 8) ^ CRCL;

CRCL=(CRC_Table[table_addr] & 0x00FF);

}

//------------------------------------------------------------------------

void Reset_CRC(void){

CRCL=0xFF;

CRCH=0xFF;

}