# Innovative Technology Ltd

# DA1 Kit

# Quick Start Guide.

## Contents

Contents	2

1. Introduction	3

2. Kit Components.	3

3. Connecting the DA1 to test a validator.	3

4. Connecting a validator to a PC using the DA1.	4

5. Software Installation	5

6. Revision History.	5

## 1. Introduction

The DA1 Kit is designed to be used for the following:

- Connection of any ITL note validator to a PC, to allow note data and firmware to be upgraded.

- Allow testing of a NV2, NV3, NV4, NV4X or NV5 note validators independently of the host machine.

## 2. Kit Components.

The DA1 Kit comprises of the following components:

  - DA1 adapter board.

  - Adapter to Validator cable (for NV2, NV3, NV4, NV4X and NV5).

  - Power Cable.

  - Quick Start Guide.

  - ITL Support CD-ROM (optional).

Please check that you have received all the parts.

## 3. Connecting the DA1 to test a validator.

The DA1 can be used to test the acceptance of a validator independently of a host machine.  This is useful for determining if the validator is faulty or if the host machine is inhibiting some of the channels.  The connections are made as shown in figure 1.  The cable with a 15-way connector on one end and a 5-way connector on the other is used to connect the validator to the DA1.  The cable with a 3.5mm jack plug and 2 banana plugs is used to supply power to the DA1.  Connect +12 volts to the red banana plug and GND (0V) to the black plug.  When using the DA1 to test note acceptance it should not be connected to a PC.

Figure 1 - Connecting a DA1 to a validator for testing

To test the validator insert the notes as normal, if the note is accepted the channel the note was accepted on is displayed on the display lights on the validator.  NV2 and NV3 validators will flash the LED corresponding to the channel. NV4, NV4X and NV5 validators will flash a binary pattern (as shown below) on the display lights on the validator. In this mode all channels are enabled (not inhibited).

| Number | LED DISPLAY 1 2 3 4 | Number | LED DISPLAY 1 2 3 4 |
| --- | --- | --- | --- |
| 1 |  | 8 |  |
| 2 |  | 9 |  |
| 3 |  | 10 |  |
| 4 |  | 11 |  |
| 5 |  | 12 |  |
| 6 |  | 13 |  |
| 7 |  | 14 |  |

## 4. Connecting a validator to a PC using the DA1.

The connections are made as shown in figure 2.  The cable with a 15-way connector on one end and a 5-way connector on the other is used to connect the validator to the DA1.  The cable with a 3.5mm jack plug and 2 banana plugs is used to supply power to the DA1.  Connect +12 volts to the red banana plug and GND (0V) to the black plug.  The 9-way D-type connector should be plugged directly into the serial port of the PC, taking note of the number of the serial port, as this will be required later when configuring the software. Once the connections have been made, the appropriate software for the validator you are using will need to be installed.

Figure 2 - Connecting a DA1 to a validator and PC for upgrading.

## 5. Software Installation

To install the software required to upgrade datasets and firmware, insert the optional CDROM into the CDROM drive.  An installation menu will appear, select the product that you wish to download, and then select the software you wish to use.  Follow the onscreen instruction to complete the installation.  Instructions for configuring and using the software you have installed can by found in the online help for that software.

## 6. Revision History.

| Issue | Mod. No. | Date | Modified by |
| --- | --- | --- | --- |
| 1 | Prod Rel | 17/08/2000 | P Dunlop |